Adapted from Putnam Lab Methods by T. Lindsay

Materials

- 100% acetone

- flammable safe fridge 4°

- Greiner UV-Star® 96 well plates | M3812-40EA

  - Need a plate that will not melt in acetone (quartz, glass or Cycloolefin Copolymer work)

- Microcentrifuge

- Vortex

- Pipette and tips (200ul or 1ml)

- 1.5 ml microfuge tubes

- Microplate Reader (We use )

- Gen5 Software on laptop

Day 1 Protocol

If not yet aliquoted

- Thaw the sample

- Aliquot 1mL of each sample into a 1.5ml tube

- Centrifuge the to separate the host and Symbiodiniaceae cells (speed and time depend on species and centrifuge)

- Remove and discard the supernatant, leaving the pellet of symbionts in the 1.5ml tube

- This tube can be frozen at this step for later analysis, or can move directly to the steps below.

If aliquoted

- Thaw symbiont pellets.

- Label one new tube “blank” and pipette 1 mL of 100% acetone into it. Treat this tube the same as all the other samples for the remainder of the protocol.

- Add 1 mL of 100% acetone to the pellet in the 1.5 mL microcentrifuge tube and vortex the tubes for 30 sec.

- Place the tubes in a fridge in the dark at 4°C for 24 hours.

Day 2 Protocol

- Turn on the plate reader and open the chlorophyll protocol.

- Remove the samples from the fridge and vortex the tubes for 15 sec.

- Spin the tubes down at 13,000 rpm for 3 minutes in the microcentrifuge to pellet any debris.

- Pipette 200µl of sample to duplicate wells of the 96 well plate (do this with the blank as well).

- Cary the plate to the plate reader and use it to quantify absorbance at 630, 663, and 750 nm in each well.

- Save the output files.

- Place the open tubes and plate in the hood to evaporate overnight.

Statistical analysis

- Save the data output as a csv file

- Create a csv file for the plate map

- In R, pivot these files to be long and merge them so that you have one file with the sample ID, and values for each of the three wavelengths

- Standardize for path length in 200µl of sample (path length of 0.6)

  - Need to correct for differences in path length of the volume in the 96 well plate compared to the 1cm path length of a cuvette.

- Calculate chlorophyll concentration

  - Chlorophyll a and c2 concentrations are calculated from the equations in Jeffrey and Humphrey 1975 after substracting A750nm from all measurements.

Chl a   =  (11.43 * 663adjusted)/0.6 – (0.64 * 630adjusted)/0.6

Chl c2 =  (27.09 * 630adjusted)/0.6 – (3.63 * 663adjusted)/0.6

Where “adjusted” means that the raw 750nm values of the blank acetone samples have been subtracted from each of the raw sample values.

Constants from Jeffery and Humphrey:

- This gives you ug chl/ml sample, but this value must be standardized by the total homogenate value and the surface area of the coral to get a final value in ug chl / cm2

References

1. Jeffrey, S. W. & Humphrey, G. F. New spectrophotometric equations for determining chlorophylls a, b, c1 and c2 in higher plants, algae and natural phytoplankton. Biochemie und Physiologie der Pflanzen 167, 191–194 (1975).

2. Warren, C. R. Rapid measurement of chlorophylls with a microplate reader. Journal of Plant Nutrition 31, 1321–1332 (2008).

3. See also: Synergy HTX Operating Manual, Gen5 Software Manual